Fitting instructions Ankorglaze

- Set top of ring beam level with top of rafter.

- Make a groove 5mm wide by 12mm deep in ring beam as per diagram below. Fit the glazing anchor profile into the groove along the ring beam between rafters. Insert the rubber gasket.

- Note: glazing anchor profile must only ne fitted to ring beam between rafters allowing rafter gasket to run complete length of the rafter and into the gutter.

- Cut the roof glazing end closure to the same width as the double-glazed unit. Fit to the gutter end of the glazing material.

- Position the glazed unit onto the roof allowing the roof glazing end closure to enter the retaining groove in the glazing anchor profile. Ease the glazed unit down the ensuring it is correctly locked in place.