[image: image1.png]exitex




MWK  R/2/20

Fitting Instructions

Threshold Section
1. Detach ramp from aluminium sill section. Check width between inside faces of the doorframe rebate and cut aluminium sill section to this size. 

	[image: image2.png]OUTSIDE

CUT RAMP
TO THIS LENGTH
>

-
CUT SILL
TO THIS LENGTH

INSIDE






	


2. Check width between outside faces of doorframe rebate and cut aluminium ramp to this size.

3. This sill requires an under door clearance of 20mm. If necessary, remove door and trim to allow for this. 
4. Position the assembled sill between the doorframe and mark fixing points through countersunk holes. Remove sill, drill fixing holes and insert plugs. 

5. Apply sealing compound to underside of sill and where sill comes into contact with the doorframe. Screw sill into place. 

6. Clip on aluminium ramp. 

Underdoor Seal Carrier

7. Remove rubber seals from aluminium carrier. Cut the underdoor seal carrier to the actual width of the door and apply a bead of Exitex Elast-O-Seal to the V groove and across each end. Fit the seal carrier with the upstand fitted tight to the bottom inside edge of the door. Screw into position. 

8. Cut the three seals 3mm longer than the aluminium and reinsert into grooves. 

	9. [image: image3.png]OUTSIDE







Weather Deflector

With the door closed measure across the outside faces of the doorframe rebate less 6mm for the two endcaps. Cut the deflector to this dimension. Install the endcaps into the aluminium section. Screw deflector into position.
